📌 Project Documentation

Wi-Fi Fingerprinting Indoor Localization using RSSI + KNN

1. Project Overview

This project implements an Indoor Localization System based on Wi-Fi Fingerprinting using the RSSI signal strength collected from nearby Wi-Fi Access Points.

The system estimates the user’s current position by comparing the live Wi-Fi signal readings with a previously built database called Radio Map.

2. Main Goal

The main goal of the project is:

✅ To predict the user’s indoor location (x, y coordinates) using Wi-Fi RSSI readings.

3. Main Concept (Wi-Fi Fingerprinting)

Wi-Fi Fingerprinting is divided into two main phases:

🔹 3.1 Offline Phase (Training Phase)

In this phase, we collect Wi-Fi RSSI readings at known reference points.

Steps:

- Stand at a known point (x, y).

- Scan Wi-Fi networks multiple times.

- Save RSSI values for selected Access Points (BSSID).

- Compute the average RSSI per point.

- Build the Radio Map database.

🔹 3.2 Online Phase (Localization Phase)

In this phase, we estimate the location of the user in real-time.

Steps:

- Collect multiple Wi-Fi scans at the unknown location.

- Compute average RSSI values.

- Compare the current fingerprint with the Radio Map.

- Apply KNN to find the nearest reference point.

- Output the estimated position.

🧠 4. Algorithms Used

4.1 RSSI Conversion

The Wi-Fi scanning tool provides SIGNAL in percentage (%).  We approximate RSSI in dBm using:

RSSI=(SIGNAL/2)−100RSSI = (SIGNAL / 2) - 100RSSI=(SIGNAL/2)−100

This is a common approximation used in Wi-Fi fingerprinting projects.

4.2 Euclidean Distance

To compare the current fingerprint with stored fingerprints, we use Euclidean Distance:

dist=∑(curr(bssid)−ref(bssid))2dist = \sqrt{\sum (curr(bssid) - ref(bssid))^2}dist=∑(curr(bssid)−ref(bssid))2

A smaller distance means the fingerprints are more similar.

4.3 K-Nearest Neighbors (KNN)

KNN is used to classify the current location.

Steps:

- Compute distance between current RSSI and each point in Radio Map.

- Sort distances.

- Select the K nearest points (K = 3).

- Choose the closest point as the estimated location.

Building Plan

Example For output

✅ Results Table (22 Runs)

| Run | Actual Position | Predicted (AVG) | AVG Correct? | Predicted (VOTE) | Vote Conf. | VOTE Correct? | Right Room |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | living_center | living_center | ✅ | living_center | 1.00 | ✅ | ✅ |
| 2 | living_center | living_center | ✅ | living_center | 0.60 | ✅ | ✅ |
| 3 | living_center_bottom | living_left | ❌ | living_left | 1.00 | ❌ | ✅ |
| 4 | living_center_bottom | living_left | ❌ | living_left | 1.00 | ❌ | ✅ |
| 5 | living_right | living_right | ✅ | living_left | 0.50 | ❌ | ✅ |
| 6 | living_right | M'sRoom | ❌ | M'sRoom | 0.60 | ❌ | ❌ |
| 7 | living_extreme_right | living_right | ❌ | living_right | 0.70 | ❌ | ✅ |
| 8 | living_right_extreme | living_left | ❌ | living_left | 0.70 | ❌ | ✅ |
| 9 | living_left | living_left | ✅ | living_left | 1.00 | ✅ | ✅ |
| 10 | living_left | living_left | ✅ | living_left | 1.00 | ✅ | ✅ |
| 11 | living_left_extreme | living_left | ❌ | living_left | 1.00 | ❌ | ✅ |
| 12 | living_left_extreme | living_left | ❌ | living_left | 1.00 | ❌ | ✅ |
| 13 | Ahmed'sRoomDoor | Ahmed'sRoomDoor | ✅ | Ahmed'sRoomCenter | 0.40 | ❌ | ✅ |
| 14 | Ahmed'sRoomCenter | Ahmed'sRoomCenter | ✅ | Ahmed'sRoomCenter | 0.80 | ✅ | ✅ |
| 15 | KitchenCenter | KitchenCenter | ✅ | KitchenCenter | 0.30 | ✅ | ✅ |
| 16 | KitchenCenter | Kitchen'Door | ❌ | Kitchen'Door | 1.00 | ❌ | ✅ |
| 17 | KitchenCenter | KitchenCenter | ✅ | Kitchen'Door | 0.30 | ❌ | ✅ |
| 18 | D'sRoom | D'sRoom | ✅ | D'sRoom | 1.00 | ✅ | ✅ |
| 19 | D'sRoom | D'sRoom | ✅ | M'sRoom | 0.40 | ❌ | ✅ |
| 20 | M'sRoom | KitchenCenter | ❌ | D'sRoom | 0.80 | ❌ | ❌ |
| 21 | M'sRoom | KitchenCenter | ❌ | M'sRoom | 0.50 | ✅ | ❌ |

🎯 Accuracy (22 Runs)

✅ Right Room = 18 / 21

Note: In the last four readings, the signal was unstable and weak. Some of the networks we are working with could not be detected. One network had a good signal, another was very poor, and the third one could not be found.

So, if we exclude the last four readings, the accuracy will be 16/17 =  94%

✅ AVG Accuracy Correct = 10 / 22  ✅ AVG Accuracy = 45.45%

✅ VOTE Accuracy

Correct = 8 / 22  ✅ VOTE Accuracy = 36.36%

⭐ Final Summary

| Method | Correct | Total | Accuracy |
| --- | --- | --- | --- |
| Right Room | 18 | 21 | 85.71% |
| AVG RSSI | 10 | 22 | 45.45% |
| Majority Vote | 8 | 22 | 36.36% |